# Configuration from Denis.

Booster 7 x HPLD-1500

Two stages preamps 2 x HPLD-1000

2 thermistors

2 protection PDs (I suppose these two are on expansion board)

1 output PD (this one as we discussed - for emergency shut down if output power drops more than 20%)

1 pointing laser

Operational logic:

2 Protection PDs

- Back-reflection PD: Monitors back-reflected power and shuts down the system if the signal exceeds a predefined threshold.

- MO PD: Monitors the Master Oscillator (MO) signal. It should prevent system startup if the signal is below the expected level and shut down the system if the signal drops below 10% of the setpoint during operation.

1 Output PD

- This PD starts monitoring the output signal whenever at least one HPLD-1500 is enabled. It should shut down the system if the output power decreases by more than 10% from the expected value.

Common

- All PDs should have:

  - Enable/disable (ON/OFF) controls

  - Adjustable calibration parameters and threshold levels

System Shutdown Procedure

- If any PD triggers a shutdown event, the system should shut down sequentially in reverse order, from the booster stage back to the first preamplifier.

- A reasonable delay should be introduced between stages (e.g., 100 ms) to ensure a controlled shutdown process.

# Realized things

Booster 7 x HPLD-1500

Two stages preamps 2 x HPLD-1000

2 thermistors

4 PD values

1 output PD value (diff)

PD levels added in config

HPLDs currents added in config
therm parameters added in config